Mid Term Test

CS 602/CO434  Image Processing

6 October, 2016

Marks: 40		Time: 60 minutes

- Fill up the blanks:								         1x5=5

a) Median filter causes less blurring than _____ filter.

b) Image sharpening technique highlights intensity _____.

c) The process used to correct the power-law response phenomena of a display device is called _____.

d) A histogram of an image is normalized by dividing each of its components by the _____.

e) Sum of weights in sharpening filters is always _____.

- State true or false:								         1x5=5

a) Sobel operator is based on 2nd order derivative.

b) A frequency domain filters has the same  size as the equivalent spatial domain filter.

c) While filtering, the center of the filter visits each pixel in the input image.

d) An image with an almost flat histogram covering the entire range is a high contrast image.

e) A larger smoothing filter will cause less blurring than a smaller one.

- Provide very brief answers for the following:					         1x5=5

a) What will happen if you apply the box filter to an image with constant gray level value?

b) Present the filter masks for the Sobel filter.

c) How do you compute first order derivative in x-direction at location (x, y) of a digital image f?

d) What is the difference between correlation and convolution?

e)  Define power spectrum of a Fourier spectrum

- In reference to digital image processing briefly answer the following:		       2x5=10

- Present a method for histogram equalization.

- How do you compute the discrete Fourier transform of an image and its inverse?

- What is unsharp masking?

- What is the dc component of the Fourier transform?

- Differentiate between spatial domain processing and frequency domain processing.

- a) Derive a sharpening filter using the second order derivative.	          			     4

b) How do you get sharpened images using the filter?					     2

- Sharpen the following image using 3x3 filters as indicated below. Show the result for the pixel at the central location only.

| 1 | 2 | 2 |
| --- | --- | --- |
| 3 | 5 | 10 |
| 4 | 6 | 6 |

- Apply the Laplacian operator with central value -4.					     3

- Apply Sobel operator.								     3

- Apply unsharp masking method to sharpen the image				     3

-x-