Midterm Exam, 19th  March 2018

CS 621: Mobile Computing

Max. Marks: 40							 	           		Time: 75 mins.

- Distinguish the following								       	          5×3=15

  - Hidden Terminal problem  vs Exposed Terminal problem

  - Reporting Cell based Location Management vs Movement based Location Management

  - Guard Channels   vs  Queuing Handoff Calls

  - Mobile Assisted Handoff vs. Network Controlled Handoff

  - Sequential Paging vs. Group Sequential Paging

- Suppose there are two wireless stations A and B which are under the access points AP1 and AP2 respectively in a WLAN setup.  The access points are interconnected using a Distribution system to form an ESS. Now, if A generates a frame to send to B, how many frames are sent in the MAC level and what are the values of 4 address fields in each of the frames?					         	            1+3=4

- Briefly explain how power saving mode for IEEE 802.11 (infrastructure based) works.		       4

- Briefly discuss about the different components and their functions in GSM architecture. How do you distinguish IMEI from IMSI?		       						            4+1=5

- Why is CSMA/CD not applicable in Wireless LAN?			       				       4

P.T.O

Column A

- IEEE 802.11ac

- Handoff  calls

- Real-time Service

- Profile based location Management

- Virtual Carrier Sensing

- Pointer Forwarding

- Asynchronous Service

- Passive Scanning

Match the entries in Column A with the most appropriate  entries in Column B 		    	     8

Column B

- Individualized dynamic approach

- DCF

- NAV

- CDMA

- MU-MIMO

- Beacon frame

- Forced termination probability

- HLR update cost

- PCF